McfTiff16 Operation Guide

December 13, 2007

NAC Image Technology Inc.

- Introduction

The McfTiff16 makes the 8 or 16 bits monochrome TIFF as the Bayer pixel array from the MCFF.

- Screen Layout of the McfTiff16

Menu Bar

Tool Bar

Dialog Bar

AOI Rectangular

Image Information on the Status Bar

- Image Information on the Status Bar

Here displays the current frame counter, the average value(s) of pixel data at cursor, pixel address of cursor and pixel data at cursor as following.

- For MCFF of the Monochrome Camera

Pixel Address at Cursor

Frame Counter  Pixel Average Value of Gray in AOI                      Pixel Gray Value at Cursor

- For MCFF of the Color Camera

Pixel Address at Cursor

Frame Counter  Pixel Average Values of Each Color in AOI               Pixel Color Value at Cursor

Above sample of the Color Camera shows non-zero value only at the Green.

This means that the pixel at cursor is the Green pixel as the MCFF is Bayer Matrix data.

- Tool Bar

Save As TIFF                            Profile

MCFF Open                       Black Balance   AOI             Zoom                        Gain

- MCFF Open	Open the file open dialog of the MCFF

- Save As MCFF	Open the file save dialog of the TIFF

The bit depth of the TIF file to be saved is specified by the Bit Depth on the Dialog Bar.

- Black Balance	Open the file open dialog of the black data MCFF

After specify the black data MCFF, it will perform the re-calibration of black level of the MCFF currently focused on the screen.

- Profile	Display the profile graph between two points on the image.

To specify two points, move cursor to the first point, press left button and drag to second point.

At this time, if the shift key is pressed, put the second point on horizontal (0 degree) or vertical (90 degree) line (it depends on the angle of dragging).

Maybe the profile is usable for the monochrome camera MCFF, since this feature doesn’t recognize either monochrome or color.

- AOI	Calculate and display the pixel average value(s) in the AOI.

To specify the rectangle, move cursor to the first point, press left button and drag to the second point.

The average value is displayed at the Pixel Average Value of the Status Bar.

If the MCFF is the data of the monochrome camera, it will display only one value.  If the MCFF is the data of the color camera, it will display three values of the Red, Green and Blue.

- Zoom	Display image with zooming.

There are 6 cases of the zooming such like as the fitting with window size, 1:1, 2x, 4x, 8x and 16x.

- Gain	Display image with the gain change by the bit shift.

This operation isn’t effective to the save TIFF data.

x1 is that takes 8 bits from MSB side of the pixel data.

For example)

When the MCFF is recorded as 12 bit, x1 takes 8 bits from 211 to 24.

x2 takes 8 bits from 210 to 23.

- Dialog Bar

All Frame

Bit Depth        Identical Frames    Start    End                  Progressive Bar

Go To Top Frame

Step Back One Frame

Stop

Playback

Step Forward One Frame

- Bit Depth	Bit depth select for the TIFF conversion

- Identical Frames	Use Start/End frame number as frame range for the conversion or playback

- Start/End	Frame range for the conversion or playback if Identical Frame is selected

- All Frame	Use all of frames in the MCFF for the conversion or playback

- Progressive Bar	Show current frame position in the frame range

- Pixel Data Format of the Saved TIFF

The McfTiff16 saves the frame data of the MCFF to the monochrome TIFF as the Bayer pixel array.

It makes the TIFF header according to the base-line TIFF standard.

When you choose 8 bits of the Bit Depth, it takes first top 8 bits of the MCFF pixels data.

When you choose 16 bits, it fills the 10 or 12 bits from LSB (see below).

MSB 215  20 LSB

16 bits

12 bits

10 bits